SECTION 1: Identification of the substance/mixture and company/undertaking

  - Product Identifier

Product Name		Super Absorbent Powder

Product Code			8229 / 8213

Size				10g   /  100g

  - Relevant identified uses of the substance or mixture and uses advised against

Identified Use		To absorb body fluid spills

  - Details of the supplier of the safety data sheet

Supplier			Steroplast Healthcare Limited

Unit 2, Alpha Point

Bradnor Road

Manchester

M22 4TE

United Kingdom

Tel: +44 (0) 161 902 3030

Email: sales@steroplast.co.uk

  - Emergency telephone number

Tel: +44 (0) 161 902 3031

SECTION 2: Hazards identification

2.1	Classification of the substance or mixture

Classification according	Mixture not classified according to CLP/GHS

to Directive 1999/45/EEC

Potential Human Health Effects

Eyes				No specific symptoms noted, but prolonged eye   					contact may lead to the product affecting the     					natural lubrication.

Skin				No specific symptoms noted, but prolonged or   					excessively repeated skin contact could lead to  					removal of natural oils from skin.

Ingestion	No specific symptoms noted, but may cause discomfort if swallowed

Inhalation			No specific symptoms noted

2.2	Label elements		Not applicable

2.3	Other hazards		Spilled product in contact with water or moisture   				causes surfaces to become slippery

SECTION 3: Composition/information on ingredients

Ingredients					Percent		CAS #

Sodium Polyacrylate Crosslinked		50-80%			9003-04-7

Dolomite carse granules			30-50%			16389-88-1

Component Information/Information on Non-Hazardous Components

The ingredients are present in non-hazardous concentrations.

SECTION 4: First aid measures

4.1	Description of first aid measures

Inhalation

If inhaled, move to source of fresh air.  Seek medical attention in case of respiratory problems

Skin Contact

Remove from skin using soap and water.

Eye Contact

Immediately flush eyes with plenty of water while lifting the eyelids.

Ingestion

No first aid measure required.

4.2	Most important symptoms and effects, both acute and delayed

See section 2.1

4.3	Indication of any immediate medical attention and special treatment needed

In case of swallowing, drink plenty of water.

SECTION 5: Firefighting measures

5.1	Extinguishing media

This product is not flammable. Use fire-extinguishing media appropriate for surrounding materials.

5.2	Special hazards arising from the substance or mixture

When heated and in case of fire, harmful vapours/gases may be formed.

5.3	Advice for fire fighters

Fire fighters should wear approved self-contained breathing apparatus

and full protective clothing.

SECTION 6: Accidental release measures

6.1	Personal precautions, protective equipment and emergency procedures

Spilled product in contact with water or moisture causes surfaces to become slippery. Avoid dust formation.

6.2	Environmental precautions

Collect and flush small residuals with much water into the drain for subsequent biological waste effluent treatment.

6.3	Methods and material for containment and cleaning up

Sweep up and shovel. Keep in suitable, closed containers for disposal.

6.4	Reference to other sections

For personal protection, see section 8.

SECTION 7: Handling and storage

7.1	Precautions for safe handling

No specific usage precautions noted.

7.2	Conditions for safe storage, including any incompatibilities

Store at moderate temperatures in dry, well ventilated area. Keep in original container.

7.3	Specific end use

Identified in Section 1.2

SECTION 8: Exposure controls/personal protection

8.1	Control Parameters

EU Limit			No information available.

UK Limit			No information available.

8.2	Exposure controls

Engineering controls	Not relevant

Eye Protection		Protective goggles.

Protective gloves		No specific hand protection noted, but protection 					for the skin is advisable to prevent removal of    					natural oils from skin.

Respirators			Respiratory protection not required

Other				None required

SECTION 9: Physical and chemical properties

9.1	Information on basic physical and chemical properties

Appearance				Off-white powder

Odour					Slight lemon

Melting/freezing point		Decomposes below the melting point

Initial boiling point/range		Not Applicable

Flash point				Not Applicable

Evaporation rate			No data available

Flammability (solid, gas)		Not Applicable

Flammability or explosive limits	Not Applicable

Vapour pressure			No data available

Relative density			2.8-2.9 g/cm3 (20°C)

Solubility (H2O)			Soluble in water

Partition coef				No data available

Auto-ignition temperature		No data available

Decomposition temperature	No data available

Viscosity				No data available

Explosive properties			No data available

Oxidising properties			No data available

9.2	Other information			Not available

SECTION 10: Stability and reactivity

10.1	Reactivity

No specific reactivity hazards associated with this product

10.2	Chemical stability

No particular stability concerns.

10.3	Possibility of hazardous reactions

See sections 10.1, 10.4 & 10.5

10.4	Conditions to avoid

None known

10.5	Incompatible materials

None noted

10.6	Hazardous decomposition products

None known

SECTION 11: Toxicological information

11.1	Information of toxicological effects

This preparation has not been tested for toxicological effects. Based on the known effects of the ingredients, the product is classified for human health effects as indicated below.

Acute toxicity

Dolomite  			LD50 Oral (Rat):>5.000 mg/kg
					Acute dermal toxicity: none

Sodium Polyacrylate		LD50 (Rat): >5,000 mg/kg

Acute dermal toxicity: LD50 (Rat):> 2000 mg/kg

Irritancy

Prolonged or repeated skin contact may cause irritation

Corrosivity

Not available

Sensitisation

Not available

Repeated dose toxicity

Not available

Carcinogeicity

Not available

Toxicity for reproduction

Not available

SECTION 12:	Ecological information

12.1	Toxicity

There us bi aquatic toxicity data specifically for this product.  However, where provided by the raw material manufacturer for ingredients with aquatic toxicity, can be made available on request.

12.2	Persistence and degradability

OECD 302 B practically no degradation

12.3	Bioaccumulative potential

The product does not contain any substances expected to be bioaccumulating

12.4	Mobility soil

Not known

12.5	Results of PBT and vPVB assessment

This product does not contain any PBT or vPvB substances

12.6	Other adverse effects

Not known

SECTION 13: Disposal considerations

13.1	Waste treatment methods

Product: dispose of whilst observing local regulations.

Unclean packaging: may be reused after proper cleaning.

SECTION 14: Transport information

This product is not transport regulated.

SECTION 15: Regulatory information

15.1	Safety, health and environmental regulations/legislation specific for the mixture

Not Applicable

15.2	Chemical safety assessment

Not Applicable

SECTION 16: Other information

Revisions

Currently in 2nd version

Updated to new format according to (EU) No 2020/878.

Disclaimer: This information summarises our best knowledge of the health and safety

Hazard information of the product and how to safely handle and use the product. Each user should read this data sheet and consider the information in the context of how the product will be handled and used including in conjunction with other products. If clarification or further information is needed to ensure that an appropriate risk assessment can be made, the user should contact Steroplast Healthcare Limited.

We assume no legal responsibility for the use or reliance upon this information.