SECTION 1: Identification of the substance/mixture and company/undertaking

  - Product Identifier

Product Name		Steroclenz Liquid / Foam
Product Code			STZ001 & STZ002

  - Relevant identified uses of the substance or mixture and uses advised against

Identified Use		Use for professional and industrial cleaning and 				disinfection purposes only. Not for domestic use. 				Not to be used on medical equipment.  Biocidal 				products (e.g. disinfectants, pest control), PC35: 				Washing and cleaning products (including solvent 				based products)

  - Details of the supplier of the safety data sheet

Supplier			Steroplast Healthcare Limited

Unit 2, Alpha Point

Bradnor Road

Manchester

M22 4TE

United Kingdom

Tel: +44 (0) 161 902 3030

Email: sales@steroplast.co.uk

  - Emergency telephone number

Tel: +44 (0) 161 902 3031

SECTION 2: Hazards identification

2.1	Classification of the substance or mixture

Classification according	Aquatic Chronic 2: H411; Eye Dam. 1: H318; Skin

to Directive 1999/45/EEC	Irrit. 2: H315; -: EUH208

Human Health		Contains polymeric biguanide hydrochloride. May 					produce an allergic reaction. Causes skin irritation. 					Causes serious eye damage. Harmful to aquatic life 					with long lasting effects.

2.2	Label elements

Hazard statements: 	EUH208: Contains polymeric biguanide hydrochloride.

May produce an allergic reaction.

H315: Causes skin irritation

H318: Causes serious eye damage

H411: Harmful to aquatic life with long lasting effects

Signal words:	Danger

Hazard Pictograms:	GHS05: Corrosion

GHS09: Environmental

Precautionary statements:

P280: Wear protective gloves/eye protection.

P302+352: IF ON SKIN: Wash with plenty of water.

P305+351+338: IF IN EYES: Rinse cautiously with water for 		several minutes. Remove contact lenses, if present and 		easy to do. Continue rinsing.

P310: Immediately call a POISON CENTRE or doctor

P321: Specific treatment (see instructions on this label)
		P362 + P364: Take off contaminated clothing and wash it 		before reuse.

2.3	Other hazards

This product is not identified as a PBT/vPvB substance.

SECTION 3: Composition/information on ingredients

3.1	Substances

Not Applicable

3.2	Mixtures

Hazardous Ingredients:

Didecyldimethylammonium Chloride

EINECS: 230-525-2	CAS:7173-51-5

CLP Classification: Acute Tox. 4: H302; Skin Corr. 1B: H314	Percent: 1-10%

Quaternary Ammonium Compounds, Benzyl-C12-16-Alkyldimethyl, Chlorides

EINECS: 270-325-2	CAS: 68424-85-1

CLP Classification: Skin Corr. 1B: H314; Eye Dam. 1: H318; Aquatic Acute 1: 	H400; Aquatic Chronic 1: H410; Acute Tox. 4: H302		Percent: 1-10%

Coconut Monoethanolamide

CAS: 61791-08-0	CLP Classification: Eye Dam. 1: H318	Percent: <1%

C12-C14 Alkyl Dimethylamine Oxide – REACH registered number(s): 01-	2119490061-47-XXXX

EINECS: 931-292-6	CAS: 308062-28-4

CLP Classification: Skin Irrit. 2: H315; Eye Dam. 1: H318; Aquatic Acute 1: H400

Aquatic Chronic 2: H411						Percent: <1%

Polymeric Biguanide Hydrochloride

EINECS: 608-042-7	CAS: 27083-27-8

CLP Classification: Acute Tox. 4: H302; Eye Dam. 1: H318; Skin Sens. 1B: H317; 	Carc. 2: H351; STOT RE 1: H372; Aquatic Acute 1: H400; Aquatic Chronic 1: 	H410; Acute Tox. 2: H330						Percent: <1%

SECTION 4: First aid measures

4.1	Description of first aid measures

Inhalation

Remove casualty from exposure ensuring one’s own safety whilst doing so.

Skin Contact

Remove all contaminated clothes and footwear immediately unless stuck to

skin. Wash immediately with plenty of soap and water.

Eye Contact

Bathe the eye with running water for 15 minutes. Transfer to hospital for

specialist examination.

Ingestion

Wash out mouth with water. Do not induce vomiting. If conscious, give half a

litre of water to drink immediately. Consult a doctor.

4.2	Most important symptoms and effects, both acute and delayed

Eye Contact: There may be pain and redness. The eyes may water profusely. 	There may be severe pain. The vision may become blurred. May cause 	permanent damage.

Skin Contact: There may be irritation and redness at the site of contact.

Inhalation: There may be irritation of the throat with a feeling of tightness in

the chest.

Ingestion: There may be soreness and redness of the mouth and throat.

Nausea and stomach pain may occur.

Delayed / immediate effects: Immediate effects can be expected after short

term exposure.

4.3	Indication of any immediate medical attention and special treatment needed

Eye bathing equipment should be available on the premises

SECTION 5: Firefighting measures

5.1	Extinguishing media

Suitable extinguisher media for the surrounding fire should be used. Use water spray to cool containers.

5.2	Special hazards arising from the substance or mixture

In combustion emits toxic fumes.

5.3	Advice for fire fighters

Fire fighters should wear and approved self-contained breathing apparatus

and full protective clothing, to prevent contact with skin and eyes.

SECTION 6: Accidental release measures

6.1	Personal precautions, protective equipment and emergency procedures

Mark out the contaminated area with signs and prevent access to unauthorised personnel. Do not attempt to take action without suitable protective clothing – see section 8 of SDS. Turn leaking containers leak-side up to prevent the escape of liquid.

6.2	Environmental precautions

Do not discharge into drains or rivers. Contain the spillage using bunding.

6.3	Methods and material for containment and cleaning up

Absorb into dry earth or sand. Transfer to a closable, labelled salvage container for disposal by an appropriate method.

6.4	Reference to other sections

Refer to section 8 of SDS.

SECTION 7: Handling and storage

7.1	Precautions for safe handling

Avoid direct contact with the substance. Ensure there is sufficient ventilation of the area. Avoid the formation or spread of mists in the air.

7.2	Conditions for safe storage, including any incompatibilities

Store in a cool, dry and well ventilated area. Keep container tightly closed. Must only be kept in original packaging.

7.3	Specific end use

No data available

SECTION 8: Exposure controls/personal protection

8.1	Control Parameters

Workplace exposure limits:		No data available
DNEL/PNEC:				No data available

8.2	Exposure controls

Engineering measures: Ensure there is sufficient ventilation of the area.

Respiratory protection: Self-contained breathing apparatus must be available in case of emergency.

Hand Protection: Protective gloves.

Eye Protection: Tightly fitting safety goggles. Ensure eye bath is to hand.

Skin Protection: Protective clothing.

SECTION 9: Physical and chemical properties

9.1	Information on basic physical and chemical properties

Appearance			Liquid, Pale yellow

Odour				Barely perceptible odour

Oxidising			Non-oxidising (by EC criteria)

Solubility in water		Soluble

Boiling Point oC		100

pH				6

Relative density		1

VOC g/l			<10

9.2	Other information		No data available

SECTION 10: Stability and reactivity

10.1	Reactivity

Stable under recommended transport or storage conditions.

10.2	Chemical stability

Stable under normal conditions.

10.3	Possibility of hazardous reactions

Hazardous reactions will not occur under normal transport or storage

Conditions. Decomposition may occur on exposure to conditions or materials listed below.

10.4	Conditions to avoid

Heat.

10.5	Incompatible materials

Avoid contact with strong oxidising agents. Strong acids.

10.6	Hazardous decomposition products

In combustion emits toxic fumes.

SECTION 11: Toxicological information

11.1	Information of toxicological effects

Hazardous ingredients

Didecyldimethlammonium Chloride

DERMAL	RAT	LD50	1300	mg/kg

DERMAL	RBT	LD50	3342	mg/kg

IPR		RAT	LD50	45	mg/kg

ORAL		RAT	LD50	238	mg/kg

ORL		MUS	LD50	268	mg/kg

C12-C14 Alkyl Dimethylamine Oxide

ORAL		RAT	LD50	1064	mg/kg

Relevant Hazards for product:

Hazard				Route		Basis

Skin corrosion/irritation		DRM		Hazardous: calculated

Serious eye damage/irritation	OPT		Hazardous: calculated

Symptoms / routes of exposure

Skin Contact: There may be irritation and redness at the site of contact.

Eye Contact: There may be pain and redness. The eyes may water profusely. There may be severe pain. The vision may become blurred. May cause permanent damage.

Ingestion: There may be soreness and redness of the mouth and throat. Nausea and stomach pain may occur.

Inhalation: There may be irritation of the throat with a feeling of tightness in the chest.

Delayed / immediate effects: Immediate effects can be expected after short-term exposure.

Other Information: Skin patch tested – no adverse reactions.

SECTION 12:	Ecological information

12.1	Toxicity

Hazardous ingredients:

DIDECYLDIMETHYLAMMONIUM CHLORIDE

DAPHNIA				48H EC50		0.03		mg/l

DAPHNIA				CE50			0.011-0.099	mg/l

DAPHNIA				NOEC			0.011-0.099	mg/l

GREEN ALGA

(Selenastrum capricornutum)		96H LC50		0.06		mg/l

ZEBRAFISH

(Brachydanio rerio)			96H LC50		0.5		mg/l

C12-C14 ALKYL DIMETHYLAMINE OXIDE

ALGAE					72H IC50		0.143		mg/l

ALGAE					NOEC			0.067		mg/l

Daphnia magna			48H EC50		3.1		mg/l

FISH					96H LC50		2.67		mg/l

12.2	Persistence and degradability

Biodegradable.

12.3	Bioaccumulative potential

No bioaccumulation potential.

12.4	Mobility soil

Readily absorbed into soil.

12.5	Results of PBT and vPVP assessment

This product is not identified as a PBT/vPvB substance.

12.6	Other adverse effects

Negligible ecotoxicity.

SECTION 13: Disposal considerations

13.1	Waste treatment methods

Disposal operations: Transfer to a suitable container and arrange for collection by specialised disposal company.

Recovery operations: Not applicable.

Waste code number: 06 13 01

Disposable of packaging: May be reused following decontamination.

NB: The user’s attention is drawn to the possible existence of regional or national regulations regarding disposal.

SECTION 14: Transport information

This product does not require a classification for transport.

SECTION 15: Regulatory information

15.1	Safety, health and environmental regulations/legislation specific for the mixture

DA PR-4260513

15.2	Chemical safety assessment

A chemical safety assessment has not been carried out for the substance or 	the mixture by the supplier.

SECTION 16: Other information

Revisions

Currently in second version

Phrases used in s.2 and s.3:

EUH208: Contains <name of sensitising substance>. May produce an allergic reaction

H302: Harmful if swallowed

H314: Causes severe skin burns and eye damage

H315: Causes skin irritation

H317: May cause an allergic skin reaction

H318: Causes serious eye damage

H330: Fatal if inhaled

H351: Suspected of causing cancer <state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard>

H372: Causes damage to organs <or state all organs effected, if known> through prolonged or repeated exposure <state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard>

H400: Very toxic to aquatic life

H410: Very toxic to aquatic life with long lasting effects

H411: Toxic to aquatic life with long lasting effects

Shelf Life

2 Years from date of Manufacture

Disclaimer: This information summarises our best knowledge of the health and safety

Hazard information of the product and how to safely handle and use the product. Each user should read this data sheet and consider the information in the context of how the product will be handled and used including in conjunction with other products. If clarification or further information is needed to ensure that an appropriate risk assessment can be made, the user should contact Steroplast Healthcare Limited.

We assume no legal responsibility for the use or reliance upon this information.